CRTC INTERCONNECTION STEERING COMMITTEE

TIF REPORT

Date Submitted:             DD MMMM YYYY

WORKING GROUP:	CSCN

REPORT #:               160A                                                           File ID:  CNRE160A

REPORT TITLE:  TBP Readiness Affirmation Form and Recommendations to Facilitate the Rollout of Thousands-Block Pooling

OUTCOME: CONSENSUS/NON-CONSENSUS

RELATED TASK(s) #:  117, 118, 119, 120, 125

BACKGROUND:

To facilitate the phased implementation of Thousands-Block Pooling (TBP), the CSCN considered what measures would be required to protect the Canadian PSTN.

FACTORS:

All carriers need to be performing LNP database queries on a databases that are configured to receive Thousands-Block records prior to the rollout of TBP in the first Exchange Areas. Carriers are also required to complete production-level testing prior to the launch of Thousands-Block Pooling in order to prevent an uptick in double assignments of Telephone Numbers (TNs) to different customers as was observed in the US when they first launched.

ALTERNATIVES:

Delay the launch of TBP in each Exchange Area where there are 1 or more carriers that are not ready for TBP. This alternative risks extending rollout of TBP for a prolonged period.

ANALYSIS:

Item 1: In Telecom Decision CRTC 2025-321 the Commission directed the Canadian Steering Committee on Numbering to create a schedule for a phased-in implementation of TBP, by Exchange Area, with the first Exchange Area transitioning to TBP by 28 July 2026, and the final exchange by 28 July 2027.  TSPs that do not operate as a LEC or Wireless Carrier in Exchange Areas where TBP rolls out may still need to perform LNP queries to terminate traffic into these Exchange Areas.  Failure to perform an LNP query on a TBP compatible database will result in mis-routed calls which will negatively impact both the TSPs customers and the terminating carrier who may be called upon by their customers to track down the routing issues.

It is impractical to determine and survey all TSPs that may terminate traffic in the rollout Exchange Areas across Canada to seek an affirmation of readiness for TBP.   A broad direction/reminder to all TSPs is the preferred route.

Additionally, we note that a number of carriers and other non-carrier entities offer competitive LNP query services. In the Essential Services decision (2008-17), the Commission classified the wholesale LNP database services of the large ILECs and Cable carriers as non-essential subject to phase-out, meaning that they could be provided without prior Commission rate approval, effective 3 March 2011. Also, in Telecom Decision 2013-133 the Commission found that certain wholesale local number portability database services should be forborne from rate regulation for all competitive local exchange carriers.

Accordingly, the CSCN recommends that the CRTC issue an order directing all TSPs (carriers and resellers) that performLNP database queries to ensure that they are performing such LNP database queries on a Thousands-Block Pooling (TBP) ready LNP database no later than July 28, 2026.

Item 2: A carrier that incorrectly puts TNs into inventory when a TN is ported to another carrier or incorrectly processes a Snapback message as a CO Coder holder where they are not the Thousands-Block holder may result in double TN assignments to the detriment of customers of both carriers involved.

Accordingly, the CSCN recommends that the CRTC issue a direction to the CNA directing it to deny issuing additional geographic number resources in pooled exchanges to any carrier that has not provided an affirmation in a prescribed form that affirms that they have performed sufficient production level testing to confirm that their:

- processing of Local Number Portability (LNP) Telephone Number (TN) disconnect notifications (i.e., Snapbacks) operates correctly in a TBP environment such that double TN assignments can be avoided;

- intake into inventory of a contaminated Thousands-Block assigned by the CNA will avoid placing ported TNs into inventory, thereby causing double TN assignments; and

- LNP database queries are performed on a database configured to receive Thousands-Block records.

Item 3: A carrier that incorrectly returns a Thousands-Block to the CNA for reassignment to another carrier may result in double TN assignments to the detriment of customers of both carriers involved.

Accordingly, the CSCN recommends that the CRTC issue a direction to the CNA directing it to refuse to accept the return of Thousands-Blocks in pooled Exchange Areas by any carrier that has not provided an affirmation in a prescribed form that affirms that they have performed sufficient production level testing to confirm that they have sufficient processes in place to return such blocks in accordance with the proposed Canadian Thousands-Block and CO Code Assignment Guideline.

CONCLUSIONS:

CSCN is of the view that the proposal is an efficient process in support of the launch of TBP.

RECOMMENDATIONS:

The CSCN recommends that:

- the CRTC issue an order directing all TSPs (carriers and resellers) that perform LNP database queries to ensure that they are performing such LNP database queries on a Thousands-Block Pooling (TBP) ready LNP database no later than July 28, 2026

- the CRTC issue a direction to the CNA directing it to deny issuing additional geographic number resources in pooled exchanges to any carrier that has not provided an affirmation in a prescribed form that affirms that they have performed sufficient production level testing to confirm that their:

- processing of Local Number Portability (LNP) Telephone Number (TN) disconnect notifications (i.e., Snapbacks) operates correctly in a TBP environment such that double TN assignments can be avoided;

- intake into inventory of a contaminated Thousands-Block assigned by the CNA will avoid placing ported TNs into inventory, thereby causing double TN assignments; and

- LNP database queries are performed on a database configured to receive Thousands-Block records.

- the CRTC issue a direction to the CNA directing it to refuse to accept the return of Thousands-Blocks in pooled Exchange Areas by any carrier that has not provided an affirmation in a prescribed form that affirms that they have performed sufficient production level testing to confirm that they have sufficient processes in place to return such blocks in accordance with the proposed Canadian Thousands-Block and CO Code Assignment Guideline.

ATTACHMENTS:

Carrier Affirmation of Thousands-Block Pooling Readiness form